STAR

How to build this in Packet Tracer

1

Open Cisco Packet Tracer → click New (blank workspace)

2

From the bottom panel, select End Devices → drag 4 PC-PT icons onto the workspace (PC0, PC1, PC2, PC4)

3Select Network Devices → Switches → drag 1 2950-24 switch to the center

4Select Connections (lightning bolt icon) → choose Copper Straight-Through cable

5Click each PC → select port FastEthernet0 → click Switch → select any free FastEthernet port. Repeat for all 4 PCs

6

Assign IPs: double-click each PC → Desktop tab → IP Configuration → set:
PC0: 192.168.1.1 / PC1: 192.168.1.2 / PC2: 192.168.1.3 / PC4: 192.168.1.4
Subnet mask: 255.255.255.0 for all

7

Test: PC0 → Desktop → Command Prompt → type ping 192.168.1.3 → should say Successful

RING

How to build this in Packet Tracer

1

Add 4 switches (2950-24) and 4 PCs to the workspace

2

Connect each PC to its nearest switch using Copper Straight-Through cable (PC0 → Switch2, PC1 → Switch0, PC2 → Switch3, PC3 → Switch1)

3

Connect switches in a ring using Copper Cross-Over cable:
Switch0 ↔ Switch1 ↔ Switch3 ↔ Switch2 ↔ Switch0

4

Assign IPs to all 4 PCs (192.168.1.1 through .4, subnet 255.255.255.0)

5

Test with ping between PCs — data travels around the ring through the switches

MESH

How to build this in Packet Tracer

1

Add 4 switches (2950-24) and 4 PCs to the workspace

2

Connect each PC to its switch using Copper Straight-Through

3

Connect every switch to every other switch using Crossover cables — that's 6 switch-to-switch links total:
SW0↔SW1, SW0↔SW2, SW0↔SW3, SW1↔SW2, SW1↔SW3, SW2↔SW3

4

Assign IPs (192.168.1.1–.4) and test with ping

💡 Mesh has multiple paths — if one link fails, data finds another route

BUS

How to build this in Packet Tracer

1

Add 3 switches and 3 PCs to the workspace — arrange them in a straight line

2

Connect each PC to its switch: PC0 → Switch0, PC1 → Switch1, PC2 → Switch2 using Straight-Through cable

3

Connect switches in a line: Switch0 ↔ Switch1 ↔ Switch2 using Crossover cables

4

Assign IPs: PC0=192.168.1.1, PC1=192.168.1.2, PC2=192.168.1.3 (subnet 255.255.255.0)

5

Test connectivity: open Command Prompt on PC0 → ping 192.168.1.3

EXP3: LAN – BUILD ANY NW BTWN(STAR,RING MESH) AND USE COMMANDS

ping ip_address

ipconfig

ipconfig /all

netstat

tracert ip_address

nslookup www.hello.com

arp -a

FOR WAN WITH ROUTER – IN ROUTER CLI

enable

conf t

configure terminal

interface portname

ip adddress ip subnet

hostname name

show ip route

EXP3- WAN

PC0,PC2 -10.10.0.0 – 10.10.0.2  -- LEFT ROUTER CABLE—10.10.0.0

PC3 – PC5 – 20.20.0.1 – 20.20.0.2 – RIGHT ROUTER CABLE 20.20.0.0

EXP – SUBNETING

From your output PDF the setup is:

Subnet 1 — 192.168.0.0/25

- PC0 → 192.168.0.1 / 255.255.255.128 / Gateway 192.168.0.3

- PC1 → 192.168.0.2 / 255.255.255.128 / Gateway 192.168.0.3

- Router Fa0/0 → 192.168.0.3 / 255.255.255.128

Subnet 2 — 192.168.0.128/25

- PC2 → 192.168.0.129 / 255.255.255.128 / Gateway 192.168.0.131

- PC3 → 192.168.0.130 / 255.255.255.128 / Gateway 192.168.0.131

- Router Fa0/1 → 192.168.0.131 / 255.255.255.128

OP :

ALL PCS – IP CONFIG SCREENSHOT, ROUTER IP CONFIG, CND PING

EXPT5: SWITCH

PCO – LAST, -- 192.168.1.1 TO 192.168.1.4

HUB –

SAME

EXP5 – ROUTER SWITCH – SAME AS SUBNETTING

From your output PDF the setup is:

Subnet 1 — 192.168.0.0/25

- PC0 → 192.168.0.1 / 255.255.255.128 / Gateway 192.168.0.3

- PC1 → 192.168.0.2 / 255.255.255.128 / Gateway 192.168.0.3

- Router Fa0/0 → 192.168.0.3 / 255.255.255.128

Subnet 2 — 192.168.0.128/25

- PC2 → 192.168.0.129 / 255.255.255.128 / Gateway 192.168.0.131

- PC3 → 192.168.0.130 / 255.255.255.128 / Gateway 192.168.0.131

- Router Fa0/1 → 192.168.0.131 / 255.255.255.128

OP – SIMULATION PANEL

EXP6:

DHCP WIRED:

SERVER – IP : 10.0.0.1,, DEFAULT,DNS -0.0.0.0

SERVICESS – DHCP -- > DEFAULT : 10.0.0.1  --- SAVE AND SERVICE ON

CHECK IP – ADDRESS OF ALL DEVICES AND TAKE SS AND OF SERVER

EXP 6 – WIRELESS

ROUTER – GUI  AUTOMATIC – SAVE SETTINGS

PC, LAPTOP --- PHYSICAL  OFF PC – REMOVE THEN ADD W300 ON PC FOR ALL

PC  PC WIRELESS – CONNECT DEFAULT CONNECT

PING, TAKE SS OF ROUTER , LAPTOP,PC , DHCP IP

EXP 7—WEB SERVER CONFIG, AND 9.1- HTTP CONFIG  :

WEB SERVER : IP – 192.168.1.2, DNS - 192.168.1.1 – SERVICES – INDEX HTML -EDIT - SAVE

DNS  : IP – 192.168.1.1, IP – 192.168.1.1,  SERVICES  DNS  NAME AND ADDRESS - 192.168.1.2, SAVE AND

PC – GIVE IP 192.168.1.3, - WEB BROWSER – URL/IP

OP -SC – IP OF PCS , SERVER DNS – HTTP, HTML , WEBPAGE

EXP 10:

Given by sir:

import math

def gcd(a, h):

temp = 0

while(1):

temp = a % h

if (temp == 0):

return h

a = h

h = temp

p = 3

q = 7

n = p * q

e = 2

phi = (p - 1) * (q - 1)

while (e < phi):

# e must be co-prime to phi and

# smaller than phi.

if (gcd(e, phi) == 1):

break

else:

e = e + 1

# Private key (d stands for decrypt)

# choosing d such that it satisfies

# d*e = 1 + k * totient

k = 2

d = (1 + (k * phi)) / e

# Message to be encrypted

msg = 12.0

print("Message data = ", msg)

# Encryption c = (msg ^ e) % n

c = pow(msg, e)

c = math.fmod(c, n)

print("Encrypted data = ", c)

# Decryption m = (c ^ d) % n

m = pow(c, d)

m = math.fmod(m, n)

print("Original Message Sent = ", m)

COMPRESSED :

def gcd(a,b):

while b: a,b=b,a%b

return a

p,q=3,7

n=p*q

phi=(p-1)*(q-1)

e=2

while gcd(e,phi)!=1: e+=1

d=pow(e,-1,phi)

msg=12

c=pow(msg,e,n)

m=pow(c,d,n)

print(msg,c,m)

RIP , OSPF, BGP – NETWORK – IP, CLOCK IS ALSO SAME

SERIAL2/0  PORTS ARE HORIZONTAL – IP – 10.0.0.2 AND FROM RIGHT – 10.0.0.03, LEFT CLOCK 64, RIGHT NOT SET

GO INTO RIP IN EACH – AD 10.0.0.0 AND 192.168.1.0 AND RIGHT ONES DIFF

OSPF Router 1

enable

conf t

router ospf 1

network 192.168.2.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.255 area 0

OSPF both Routers
conf t

no ip route 192.168.2.0 255.255.255.0 10.0.0.2

no ip route 192.168.0.0 255.255.255.0 10.0.0.1

end

BGP Router 0
enable

conf t

interface fastEthernet0/0

ip address 192.168.0.1 255.255.255.0

no shutdown

exit

interface fastEthernet1/0

ip address 10.0.0.1 255.255.255.0

no shutdown

exit

ip route 192.168.2.0 255.255.255.0 10.0.0.2

BGP Router 1

enable

conf t

interface fastEthernet0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface fastEthernet1/0

ip address 10.0.0.2 255.255.255.0

no shutdown

exit

ip route 192.168.0.0 255.255.255.0 10.0.0.1

FTP:

GO TO SERVER:

TURN ALL OFF EXCEPT FTP – ADD USERNAME AND PASSWORD

PC – ipconfig

ftp serverip

add id and password

dir

make text file – add and save in .html

go back to pc

put filename.html

dir

FTP

Take server0
PC 0
Switch
Server ip
192.168.0.1
Defaul Gateway
0.0.0.0
PC IP
192.168.0.2
PC Gateway
192.168.0.1
Go on server 
Services and turn off
all services
PC0 Cmd code

ipconfig
ftp 192.168.0.1

pass  123

ftp>dir

help

Go to Text editor

Save file .html

PC0 Cmd code

put ___.html

dir

telnet and ssh have same structure:

TELNET Router 0
conf t

interface fastEthernet0/0
ip address 192.168.1.1 255.255.255.0

no shutdown

exit

hostname R1

enable secret rp

line vty 0 5

password tp

login

exit

TELNET PC 0

ping 192.168.0.1

ipconfig

SSH CLI code
Router 0
PC 0
Switch 0
! R1 Configuration:

en

config t
hostname R1

int f0/0
ip add 192.168.1.1 255.255.255.0

no sh
enable password abc
ip domain-name abc
username abc password abc
crypto key generate rsa
ans--2048

ip ssh version 2

line vty 0 4
transport input ssh
login local
press ctrl z

PC0 Cmd

Ssh –l abc 192.168.1.1
Password:abc

En

Sh ip int br